# 10 Year Facilities Plan 2016-2025

Table of Contents

| Building | Page |
| --- | --- |
| A.H. Dakin School | 2-4 |
| Fulham School | 5-6 |
| Grand Trunk High School | 7-9 |
| Evergreen School | 10-11 |
| Niton Central School | 12-14 |
| Summitview School | 15-16 |
| Wildwood School | 17 |
| Crescent Valley School | 18-19 |
| École Mountain View School | 20 |
| École Pine Grove School | 21 |
| École Westhaven School | 22-23 |
| Evansview School | 24-25 |
| Harry Collinge High School | 26-27 |
| Grande Cache Community High School | 28-29 |
| Jasper Elementary School | 30-31 |
| Jasper Jr./Sr. High School | 32 |
| Parkland Composite High School | 33-34 |
| Sheldon Coates School | 35-36 |
| Wildwood, Hinton & Grande Cache Shops & Bus Compound | 37 |
| ESC, SSSC & HLSC Building | 38 |

GYPSD #77 Ten-Year Facilities Plan (2016 – 2025)

| A.H. Dakin School 4537 – 7 Avenue Edson Alberta |  |
| --- | --- |
| Grades | K – 5 |
| Enrollment (2014/2015) | 135 |
| Current Utilization (2013/2014) | 59 % |
| Alberta Infrastructure Net Capacity | 273 |
| Type of Construction | Wood Frame, 2 Storey |
| Year Constructed | 1956 – Original building – 1170 M2 1958 – Addition – Gym & North Section – 1201.2 M2 |
| Total Building Area | 2371.2 M2 |
| FCI | 27.53 % August 2011 |

Facility Status Summary Chart

| Last Facility Audit (2011) | Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- | --- |
| 2011 - Fixed Casework –Perimeter shelves are built around failing heating/ventilator units Install perimeter casework with adjustable shelving complete with doors |  |  | This facility has been approved for replacement |
| 2011 - Unit Ventilators – Replace all unit ventilators with wall fin radiation |  |  |  |
| 2011 - Waste & Vent Piping – Drainage lines show signs of corrosion and there is evidence of leaks |  |  |  |
| 2011 - Sheet Metal Roofing – Metal sloped roof cladding at exit adjacent gymnasium |  |  |  |
| 2011 - Roofing – Built up roofing over the mud roof, and the 1956 and 1958 sections of the building |  |  |  |
| Exterior Stairs – modify existing, cast new concrete stairs | 2006 – Replaced steps & sidewalk (IMR) |  |  |
| Vinyl Siding – Install Cladding trims & accessories | Repaired – General maintenance |  |  |
| Cement Plaster (Stucco)- Repair window infill’s |  |  |  |
| Joint Sealers – Caulk windows | 2009 Replace caulking (P.O. & M) |  |  |
| Sheet Metal Roofing – Repair gym roof |  |  |  |
| Interior Fixed Partitions – Re-clad partitions |  |  |  |
| Interior Swinging Doors – Re-place doors |  |  |  |
| Visual Display Boards – Install new chalkboards | Chalkboards replaced with Smartboard Installs |  |  |
| Handrails – Modify handrails to comply with ABC |  |  |  |
| Other Fittings – Install new lockers | Other Fittings – Install new lockers |  |  |
| Resilient Stair Finishes- Replace stair treads | Resilient Stair Finishes- Replace stair treads |  |  |
| Paint Concrete Floor Finishes in Janitor and Mechanical Rms – Repaint concrete floors |  |  |  |
| Carpet Flooring in Admin., Library & Music Rms – replace carpet flooring & carpet bases | 2008 Admin. & upstairs Classroom Flooring Replaced (IMR) 2009 – Replace lower Classroom Flooring (IMR) 2010 – Hallway flooring replaced |  |  |
| Acoustic Ceiling Treatment (Susp.T-bar)-replace damaged ceiling tiles | 2007 – Replaced Tiles (P.O. & M) |  |  |
| Interior Ceiling Painting- Washrooms, Staircases & Service Rms - Paint finishes are dull & damaged in areas | 2008 – Painting upstairs Classrooms & Hallways (P.O. & M) 2009 – Paint Lower Floor Classrooms & Hallways (P.O. & M) |  |  |
| Urinals – Floor mounted urinals, flush valves |  |  |  |
| Electrical Branch Wiring – present number of receptacles inside each classroom are not enough to meet present load requirements – add two receptacles in each classroom | Receptacles being added during Smartboard Installs |  |  |
| Fixed Casework – Install perimeter casework with adjustable shelving with doors in classrooms |  |  |  |
| Barrier Free Entrances – Install 18 linear meters of ramp(s) & steel railing | 2012 – Install barrier free ramp and railing (IMR) |  |  |
| Barrier Free Interior Circulation – Incorporate an elevator for barrier free access | 2012 – Install Stair Climber (IMR) |  |  |
| Athletic & Recreational Surfaces–repair cracks in the asphalt play surface. | 2007 – Replaced (IMR) |  |  |
| Flagpoles – Install new flag poles | 2007 – Replaced Flagpoles (IMR) |  |  |

a)	Enrollment Pressures/Learning Opportunities:

Expected growth in the east end of Edson could result in increased enrollment of this facility.

| 3 Year Projection |  |  |  |
| --- | --- | --- | --- |
| Year | 15-16 | 16-17 | 17-18 |
| Gr.K-5 | 151 | 157 | 161 |

b)	Program Changes:  none anticipated

c)	Enrollment History

| 8 Year Historical |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | 07-08 | 08-09 | 09-10 | 10-11 | 11-12 | 12-13 | 13-14 | 14-15 |
| Gr.K-5 | 159 | 141 | 111 | 116 | 136 | 157 | 140 | 135 |

| Fulham School RR #1 Peers Alberta |  |
| --- | --- |
| Grades | K - 6 |
| Enrollment (2014/2015) | 90 |
| Current Utilization (2013/2014) | 66 % |
| Alberta Infrastructure Net Capacity | 138 |
| Type of Construction | Wood Frame |
| Year Constructed | 1956 – Original building – 540 M2 1959 – Addition – 232.9 M2 2000 – Addition Library & 4 Portables – 537.77 M2 |
| Total Building Area | 1, 310.67 M2 |
| FCI | 24.50 % August 2011 |

Facility Status Summary Chart

| Last Facility Audit (2006) | Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- | --- |
| 2011 – Replace fluorescent fixtures with T8 lamps |  | 2015 – Replace Septic Tank (IMR 14-15) | Replace existing portables with new modulars as identified in GYPSD’s 3 year Capital Plan |
| 2011 – Replace furnaces with Air Handling Units (2) (1956 & 1959 sections) |  | 2015 – Re-roof (IMR 14-15) |  |
| 2011 – Upgrade insulation & install SBS roofing (1956 & 1959 sections) |  | 2015 – Replace furnace & install DDC (IMR 14-15) |  |
| 2011 - Wood Windows (Glass & Frame) in washrooms & vestibule – Install new windows |  | 2015 – Install Cardlock (IMR 14-15) |  |
| Plaster Wall Finishes – Install wall tiles in washrooms | 2009 – Washroom Upgrade (IMR) |  |  |
| Tile Floor Finishes – Install resilient flooring in corridor | 2008 – Flooring replaced (P.O. & M) 2011 – Replace flooring in 3 classrooms |  |  |
| Resilient Flooring – in corridor & classrooms of VCT – Install resilient flooring | 2008 – Corridor Flooring replaced (P.O. & M) 2012 – Refinish Gym Floor (IMR 11-12) |  |  |
| Carpet Flooring – Install carpet | 2008 – Admin. Area replaced as part of office Reno |  |  |
| Furnaces – 1956 Section – Replace two furnace units |  |  |  |
| Furnaces – 1959 Section - Flame Master furnace for Gym – replace furnace |  |  |  |
| Exit Signs – Replace exit signs with new |  |  |  |
| Fixed Casework – Original plywood casework – Replace casework |  |  |  |
| Curtains & Drapes – in Admin. Spaces – Replace curtains | 2005 – Replaced during Office Reno (IMR) |  |  |
| Aggregate Roadway (Gravel) – Pave roadway | 2006 – Paved (IMR) 2013 – Replace Sidewalk |  |  |
| Flagpoles – Install Flagpoles | 2007 – Replaced Flagpoles (IMR) |  |  |
|  | 2007 – Portables Re-roofed (IMR) |  |  |
|  | 2008 – A New Well was drilled (IMR) |  |  |
|  | 2008 – P.A. System installed (IMR) |  |  |
|  | 2010 – Install T-bar ceiling and new lighting in portables 2010 – Upgrade kitchen |  |  |
|  | 2014 – Replace Fire Alarm Panel (IMR 13-14) |  |  |

a)	Enrollment Pressures / Learning Opportunities:  none anticipated

| 3 Year Projection |  |  |  |
| --- | --- | --- | --- |
| Year | 15-16 | 16-17 | 17-18 |
| Gr.K-6 | 87 | 84 | 91 |

b)	Program Changes:  none anticipated

c)	Enrollment History

| 8 Year Historical |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | 07-08 | 08-09 | 09-10 | 10-11 | 11-12 | 12-13 | 13-14 | 14-15 |
| Gr.K-6 | 95 | 85 | 79 | 82 | 82 | 89 | 88 | 90 |

| Grand Trunk High School 4704 – 46 Avenue Evansburg Alberta |  |
| --- | --- |
| Grades | 7 – 12 |
| Enrollment (2014/2015) | 163 |
| Current Utilization (2013/2014) | 31 % |
| Alberta Infrastructure Net Capacity | 482 |
| Type of Construction | Masonry, one storey |
| Year Constructed | 1972 – Original building |
| Total Building Area | 4,954.70 M2 |
| FCI | 19.55 % September 2011 |

Facility Status Summary Chart

| Last Facility Audit (2011) | Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- | --- |
|  | 2014 – Fire Alarm Power Supply (IMR 13-14) | 2015 – Cardlock (IMR 14-15) | Complete Modernization of this Facility as identified in GYPSD’s 3 year Capital Plan. |
| 2011 – Air Quality – Add range hoods, roof mounted exhaust fans and makeup air unit in Home Economics room |  | 2016 – Renovate front entrance to incorporate a student gathering area as identified in GYPSD’s 3 year Capital Plan | Addition to gym to accommodate bleachers as identified in GYPSD’s 3 year Capital Plan |
| 2011 – Replace fume hood and associated cabinet and install two new vented chemical storage cabinets |  |  |  |
| 2011 – Replace PA and Music System (24 stations) |  |  |  |
| 2011 – Replace emergency lighting battery packs (30 units) |  |  |  |
| 2011 – Replace Air Handling Unit 1 | 2013 - Completed |  |  |
| 2011 – Replace Water Closets & Urinals |  |  |  |
| Built-up Bituminous Roofing (Asphalt & Gravel) – Upgrade insulation/roofing | 2007 – Roof Section completed (IMR) 2009 – Roof section to be completed (IMR) 2010 – Re-roofing RA 8 2011 – Re Roof RA 4 2012 – Re Roof RA 1 (IMR 12-13) 2013 – Re-roof (IMR 12-13) 2014 – Re-roof (IMR 13-14) RA 8 & 9 |  |  |
| Lockers – Install heavy gauge metal lockers | 2007 – Lockers replaced (IMR) |  |  |
| Paint Concrete Floor Finishes – in Shops, mezzanine overlooking the gym & mechanical Rooms | 2009 - Completed |  |  |
| Wood Flooring – hardwood flooring installed in gym – repair wood floor | 2011 – New heating/ventilation in gym & CTS area. Replace concrete slab, & hardwood floor in gym |  |  |
| Carpet Flooring – replace all carpet | 2008 – Library Carpet replaced (P.O &M) |  |  |
| Air Handling Units: Air Distribution – Replace air handling units | 2009 – Replace 2 air handling units (IMR & P.O. & M) 2013 – Upgrade HVAC (IMR 12-13) |  |  |
| Fans: Exhaust – Home Economics – Install range hood |  |  |  |
| Computer Room Air Conditioning Units – no air conditioning units in both computer labs – Install computer lab A/C Units | 2009 – Install air conditioning units in computer labs (IMR & P.O. & M) |  |  |
| Motor Starters & Accessories – disconnect switch is inaccessible – DDC system installed in 1989 – Relocate motor disconnect switch | 2009 - Completed |  |  |
| Interior Fluorescent Fixtures – school uses T12 fluorescent fixtures – Replace T12 fixtures with T8 fixtures |  |  |  |
| Exit Signs – Existing Exits signs are with incandescent bulbs – Replace exit signs |  |  |  |
| Video Surveillance – no video system in school – Install a Video Surveillance system | 2009 – Video surveillance to be installed (IMR & School budget) |  |  |
| Public Address & Music System – Replacement of old PA system | 2008 – Replaced (IMR) |  |  |
| Fixed Casework – Cabinet doors & hardware have been removed – Upgrade fixed cabinets |  |  |  |
| Barrier Free Entrances – main building entrance door does not have an automatic opener – Upgrade door hardware |  |  |  |
| Aggregate Parking Lots (Gravel) – student & staff area parking lots – Pave Staff & Student parking lots | 2008 – Parking lots paved (IMR) |  |  |
| Exterior Steps & Ramps – Incorporate a ramp to the main entrance walkway |  |  |  |
| Flagpoles – Install new flagpoles | 2007 – new flagpoles installed |  |  |
|  | 2006 – Exterior Painted – Staff Room window replaced (IMR) |  |  |
| Area Lighting – Replace pole fixtures & replace pole lighting fixtures with new air wall mounted perimeter lighting | 2011 – Renovate CTS area into a functional music area |  |  |

Enrollment Pressures / Learning Opportunities: Declining enrollments.

| 3 Year Projection |  |  |  |
| --- | --- | --- | --- |
| Year | 15-16 | 16-17 | 17-18 |
| Gr.7-12 | 154 | 160 | 168 |

b)	Program Changes:  none anticipated

c)	Enrollment History

| 8 Year Historical |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | 07-08 | 08-09 | 09-10 | 10-11 | 11-12 | 12-13 | 13-14 | 14-15 |
| Gr.7-12 | 233 | 207 | 190 | 168 | 152 | 144 | 148 | 163 |

| Evergreen School 5312 – 5 Avenue Edson Alberta |  |
| --- | --- |
| Grades | K - 5 |
| Enrollment (2014/2015) | 259 |
| Current Utilization (2013/2014) | 116 % |
| Alberta Infrastructure Net Capacity | 220 |
| Type of Construction | Masonry |
| Year Constructed | 1960 – Original building – 1841 M2 1990 – Modernization 2002 – Portable Addition – 115.19 M2 2010 – Modus Modular Addition – 111.48 M2 |
| Total Building Area | 2,067.67 M2 |
| FCI | 9.51 % November 2012 |

Facility Status Summary Chart

| Last Facility Audit (2007) | Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- | --- |
| 2012 – Install Barrier Free Washroom |  |  | Complete modernization of this facility as identified in GYPSD’s 3 year Capital Plan |
| 2012 – Install Chair Lift for access to gym |  |  |  |
| 2012 – Barrier Free – Install 2 Power Actuated Doors |  |  |  |
| Exterior Gym Stairs- Replace stairs leading to gym with ramp extending to residential street |  |  |  |
| Barrier Free Entrance – Install barrier free access at main entry |  |  |  |
| Handicap Access to Gym – Install barrier free access to gym |  |  |  |
| Barrier Free Washroom – Install barrier free washroom equipment |  |  |  |
| Aggregate Parking Lots – Expand 200 m2 and refurbish with asphalt | 2010 – Parking lot paved |  |  |
| Asphalt Playground – Expand play area to 270 m2 and replace failed asphalt. Replace 704 m2 recreational surfaces with new asphalt | 2006 – Asphalt play area replaced (IMR) |  |  |
|  | 2009 – Replace Office & entrance Flooring (P.O. & M) |  |  |
|  | 2009 - New Modular |  |  |
|  | 2006 – Gym Re-Roofed (IMR) |  |  |
|  | 2007 – New Flooring in Classrooms (IMR) |  |  |
|  | 2007 – New Washroom Partitions (IMR) |  |  |
|  | 2007 – DDC Controls installed (IMR) |  |  |
|  | 2008 – Remainder of Classroom Flooring replaced (IMR) |  |  |
|  | 2009 – 1 new modular added |  |  |
|  | 2012 – Replace RTU (IMR12-13) |  |  |

a)	Enrollment Pressures / Learning Opportunities:

One portable classroom added in December of 2004 to relieve enrollment pressures, a second portable was added in 2010 to relieve enrollment pressures.

| 3 Year Projection |  |  |  |
| --- | --- | --- | --- |
| Year | 15-16 | 16-17 | 17-18 |
| Gr.K-5 | 256 | 256 | 251 |

b)	Program Changes:  none anticipated

c)	Enrollment History

| 8 Year Historical |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | 07-08 | 08-09 | 09-10 | 10-11 | 11-12 | 12-13 | 13-14 | 14-15 |
| Gr.K-5 | 246 | 248 | 241 | 270 | 263 | 247 | 253 | 259 |

| Niton Central School Niton Junction Alberta |  |
| --- | --- |
| Grades | K – 9 |
| Enrollment (2014/2015) | 168 |
| Current Utilization (2013/2014) | 47% |
| Alberta Infrastructure Net Capacity | 394 |
| Type of Construction | 1952/56/72 – Wood Frame 1960 – Masonry 1985 – Pre-cast concrete |
| Year Constructed | 1952 – Original building 585.2 M2 1956 / 58 – Addition – 1531.7 M2 1960 / 72 – Addition – 748.80 M2 1985 – Modernization & addition of gymnasium & administration area – 1010.1 M2 |
| Total Building Area | 3,875.80 M2 |
| FCI | 17.38 % September 2011 |

Facility Status Summary Chart

| Last Facility Audit (2011) | Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- | --- |
| 2011 - Fire Code – No containment tray for generator diesel fuel storage. Fabricate water tight containment tray the same volume as fuel storage tank |  | 2015 – Cardlock (IMR 14-15) |  |
| 2011 - Indoor Environment – No BMCS. Install a BMCS | 2013 – Install DDC Completed (IMR 12-13) |  |  |
| Paint (&Stains) Exterior Walls – wood shakes, soffits & wood siding – repair & refinish exterior wood | Completed in Painting Program (P.O. & M) |  |  |
| Metal Gutters & Down –spouts – replace gutters & downspouts | 2010 -replaced during roofing replacement |  |  |
|  | 2014 – Heat Trace wiring (IMR 13-14) |  |  |
| Fabricated Compartments (Toilets/Showers) – replace metal partitions | 2008 – All washrooms upgraded (IMR) |  |  |
| Wood Flooring – Gym – refinish hardwood floor | 2008 – Gym Floor refinished (IMR) |  |  |
| Resilient Flooring – replace resilient flooring | 2008 – Elementary hallway replaced (IMR) 2009 – Replace Flooring in Main entrance & hallway 2010 – Hallway flooring replaced 2010 – Library floor repaired 2014 – Flooring Hallway, room 122, 134, 202 |  |  |
| Interior Ceiling Painting – Dressing Rms, showers, washrooms & service Rms – finishes appear dull & in need or refreshing | 2006 – Shower Rooms refinished (IMR) 2011 – Renovate office area 2012 – Four classroom upstairs to have new flooring, millwork & paint (IMR 11-12) |  |  |
| Area Drains – 3 Sump Pumps installed around basement Mechanical Rm – replace sump pumps | 2009 – Foundation repair (Capital Funding) |  |  |
| Air Distribution – replace existing ductwork |  |  | As Identified in GYPSD’s 3 year Capital Plan |
| Electrical Branch Wiring – Existing number of Receptacles in each classroom not enough for present load requirement – add two receptacles in each classroom | Additional plugs added with Smartboard Installs |  |  |
| Interior Fluorescent Fixtures – the school has existing T12 fixtures – upgrade to T8 fixtures | 2008 – Elementary hallway fixtures upgraded (IMR) |  |  |
| Interior Metal Halide Fixture – gym has 40-250W Metal halide fixtures half of which are turned ‘Off because of ballast noise – replace each ballast on metal halide fixtures | 2007 – Gym Lights replaced (IMR) |  |  |
| Public Address & Music System – replace old PA system | 2011 - Replaced |  |  |
| Site Earthwork (Site Grading) – Poor Drainage issues remain along east side of 1960’s structure | 2009 – New weeping tile (Capital Funding) 2009 - Completed |  |  |
| Aggregate Roadway (Gravel) – Bus drop-off, fire lane & access to parking is gravel – Pave roadways | 2010 – Bus lane paved |  |  |
| Aggregate Parking Lots (Gravel) – Staff, visitor & students – Pave parking areas | 2010 – Staff parking lot paved |  |  |
| Rigid Pedestrian Pavement (Concrete) – walkways at west & north side of building – Concrete plaza area for pool. Remove & install new sidewalks | 2007 – Sidewalks replaced (IMR) |  |  |
| Flagpoles – Install steel poles | 2007 – Flagpoles replaced (IMR) |  |  |
| Install Elevator / Lift & build enclosed vestibule to allow one stair from the second floor direct access to the exterior | 2009 – Elevator installed |  | As identified in GYPSD’s Three Year Capital Plan |
| Re-roofing shingle area | 2009 – ReRoofing Scheduled (IMR) 2011 – Re-Roof RA 1,2 & 4 plus shingles 2013 – Re-roof (IMR 12-13) 2014 – Re-roof RA 7 & 8 (IMR 13-14) |  |  |

a)	Enrollment Pressures / Learning Opportunities:  none anticipated

| 3 Year Projection |  |  |  |
| --- | --- | --- | --- |
| Year | 15-16 | 16-17 | 17-18 |
| Gr.K-9 | 166 | 168 | 161 |

b)	Program Changes:  none anticipated

- Enrollment History

| 8 Year Historical |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | 07-08 | 08-09 | 09-10 | 10-11 | 11-12 | 12-13 | 13-14 | 14-15 |
| Gr.K-9 | 165 | 153 | 147 | 151 | 157 | 144 | 165 | 168 |

| Summitview School 10402 Hoppe Avenue Grande Cache Alberta |  |
| --- | --- |
| Grades | 4 – 8 & L.E. |
| Enrollment (2014/2015) | 219 |
| Current Utilization (2013/2014) | 58% |
| Alberta Infrastructure Net Capacity | 448 |
| Type of Construction | Masonry |
| Year Constructed | 1970 – Original building – 2577 M2 1981 – Addition – 1445.57 M2 |
| Total Building Area | 4,022.57 M2 |
| FCI | 20.47% October 2010 |

Facility Status Summary Chart

| Last Facility Audit (2010) | Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- | --- |
| 2010 – Air Handling Units – Replace Multizone Units (3) | 2012 – Replaced 1 additional | 2016 – Replace remaining units (IMR 14-15) |  |
| 2010 – Replace resilient tile flooring in classrooms |  |  |  |
|  | 2014 – Install railing at front entrance stairs |  |  |
| 2010 – Replace Exterior Joint Sealers (caulking) |  |  |  |
| 2010 - Concrete Block Ext. Wall Skin – Cladding of exterior walls with an EIFS system or insulated exterior cladding |  |  |  |
| Joint Sealers (caulking) Ext. Wall – Redo caulking on exterior of building | 2009 – Redo Caulking (P.O. & M) |  |  |
| Paints (&Stains) Ext. Walls – Re seal exterior wall areas where paint has flaked off & repaint | 2008 – Repainted (P.O. & M) |  |  |
| Exterior Wall Vapor Retarder, Air Barriers & Insulation – Study conditions of envelope |  |  |  |
| Vinyl, Fibreglass & Plastic Windows – Replace all vinyl windows in building |  |  |  |
| Built-Up Bituminous Roofing (Asphalt & Gravel) – Replace original roofs with SBS roof membranes | Several Roof Sections replaced (IMR) 2009 – additional roof section scheduled (IMR 2012 - Re-Roof RA 3-5,10,11,16-19 (IMR 11-12) |  |  |
| Interior Wall Painting – Repaint interior walls | Several areas repainted over the last few years (P.O. & M) 2009 – Continue with Painting program (P.O. & M) | 2013 - Painting |  |
| Resilient Flooring – Replace vinyl tile & rubber base | 2007/2008 – all Hallway floors replaced 2011 – Refinish gym floor |  |  |
| Carpet Flooring – Replace carpet in 3 to 5 years | 2008 – Library & Admin. Office carpet replaced (P.O. &M) |  |  |
| Interior Ceiling Painting – Reprime & paint Gym Deck above | 2008 – Gym Ceiling repainted (P.O.&M) |  |  |
| Air Handling Units: Air Distribution – Replace multizone units | 2011 – Replace 1 multi zone unit & add 1 new unit |  | Upgrade Heating & Ventilation as Identified in GYPSD’s 3 year Capital Plan |
| Other – Nonstandard custom made wood boot racks at main entry – Replace Metal bootracks to replace existing | 2008 – Replaced Boot racks (IMR) |  |  |
| Paving Parking Lots (Asphalt) | Part of Building & Grounds improvements to be completed over the next 3 years. 2009 - Parking lots paved 2009 – Install Handicap lift into gym |  |  |

a)	Enrollment Pressures / Learning Opportunities:

Expected growth in this area could result in higher enrollments.

| 3 Year Projection |  |  |  |
| --- | --- | --- | --- |
| Year | 15-16 | 16-17 | 17-18 |
| Gr.4-8 & LE | 212 | 210 | 229 |

b)	Program Changes:  none anticipated

c)	Enrollment History

| 8 Year Historical |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | 07-08 | 08-09 | 09-10 | 10-11 | 11-12 | 12-13 | 13-14 | 14-15 |
| Gr.4-8 & LE | 239 | 216 | 237 | 249 | 266 | 252 | 238 | 219 |

| Wildwood School 5311 – 47 Street Wildwood Alberta |  |
| --- | --- |
| Grades | K - 9 |
| Enrollment (2014/2015) | 127 |
| Current Utilization (2013/2014) | 46 % |
| Alberta Infrastructure Net Capacity | 321 |
| Type of Construction | One Storey Wood Frame & Masonry |
| Year Constructed | 1956 – Original building – 650.7 M2 1963 – Addition – 880.3 M2 1985 – Gym Addition & Modernization – 1414.10 M2 |
| Total Building Area | 2,945.1 M2 |
| FCI | 34.32 % September 2011 |

Facility Status Summary Chart

| Last Facility Audit (2006) | Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- | --- |
| 2011 – Replace furnaces with ventilation unit and radiation |  |  |  |
| 2011 – Repair Exterior Wall - Stucco |  |  |  |
| Ramp Finishes – Sheet Vinyl – Install resilient flooring to ramps | 2009 – Hallway Flooring replaced (IMR) |  |  |
| Furnaces – Replace furnaces with new system |  |  | As Identified in GYPSD’s 3 Year Capital Plan |
| Computer Room Air Conditioning Units – Install 5 ton air conditioners |  |  | As Identified in GYPSD’s 3 Year Capital Plan |
| Public Address & Music Systems – Replace existing PA system | 2008 – Intercom Replaced (IMR) |  |  |
| Parking Lots Signs – Install Parking lot signs |  |  |  |
| Flagpoles – Install new Aluminum flagpoles | 2007 – New Flag Poles installed (IMR) |  |  |
|  | 2009 – Gym Floor replaced |  |  |
|  | 2013 – Re-roof (IMR 12-13) 2014 – Re-roof RA 1 (IMR 13-14) | 2015 – Re-Roof (IMR 14-15) |  |

a)	Enrollment Pressures / Learning Opportunities: none anticipated

| 3 Year Projection |  |  |  |
| --- | --- | --- | --- |
| Year | 15-16 | 16-17 | 17-18 |
| Gr.K-9 | 110 | 103 | 106 |

- Program Changes: none anticipated

- Enrollment History

| 8 Year Historical |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | 07-08 | 08-09 | 09-10 | 10-11 | 11-12 | 12-13 | 13-14 | 14-15 |
| Gr.K-9 | 124 | 125 | 120 | 133 | 128 | 125 | 137 | 127 |

| Crescent Valley School 213 Tamarack Avenue Hinton Alberta |  |
| --- | --- |
| Grades | K - 7 |
| Enrollment (2014/2015) | 346, |
| Current Utilization (2013/2014) | 53 % |
| Alberta Infrastructure Net Capacity | 599 |
| Type of Construction | Masonry |
| Year Constructed | 1958 – Original building – 2775.80 M2 1966 – Two-storey Addition & Renovation – 2,199.7 M2 2001 - Modernization |
| Total Building Area | 4, 975.50 M2 |
| FCI | 18.66 % October 2010 |

Facility Status Summary Chart

| Last Facility Audit (2010) | Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- | --- |
| 2010 – Replace SBS Roofing (1958 structure) |  | 2015 – Install new lockers |  |
| Joint Sealers (caulking) Ext. Wall – Redo all exterior caulking on exterior of school | 2008 – Repaired caulking (P.O. & M) 2011 – Re-roof RA 4 &5 2014 – Reinsulate gym roof | 2015 – Re-roof (IMR 14-15) |  |
| Paints (&Stains) Ext. Wall – Repaint exterior wood surfaces & metal cap flashings | 2008 – Painted Exterior (P.O. & M) 2014 – Install Cardlock access (IMR 13-14) |  |  |
| Other Exterior Wall Skin – Replace damaged exterior spandrel panels | 2008 – Replaced & painted (P.O. & M) |  |  |
| Exterior Louvers, Grilles & Screens – Replace steel louvre with custom aluminum louvre | 2008 – Replaced Grilles & Louvers (IMR) |  |  |
| Aluminum Windows (Glass & Frame) – Replace original windows in 1966 section with new windows |  |  |  |
| Other Exterior Windows – Replace wood windows in building |  |  |  |
| Storage Shelving – Provide new shelving units in classrooms | Replaced Shelving (IMR) |  |  |
| Resilient Flooring – Cover existing V.A. Tile with sheet vinyl flooring in classrooms | Replaced Flooring (IMR) 2014 – Flooring Library & AV (IMR 13-14) |  |  |
| Urinals – Provide flushometers |  |  |  |
| Computer Room Air Conditioning Units – Provide air-conditioning in the computer lab |  |  |  |
| Building Systems Controls (BMCS, EMCS) – Expand DDC Controls | 2008 – Installed DDC Controls (IMR) |  |  |
| Call Systems – Replace call system | 2008 – Replaced intercom (IMR) |  |  |
|  | 2001 – Major Reno of 1966 building area using One-time Block Funding |  |  |
|  | 2004 – All Washrooms upgraded (IMR) |  |  |
|  | 2006 – All Classrooms& wheelchair ramp were upgraded (IMR) |  |  |
|  | 2008 – Office & Admin. Area reconfigured (IMR) |  |  |
|  | 2012 – Clean AHU, RTU and Ducts (IMR 12-12) |  |  |
|  | 2012 – Replace failed gas line 2014 – Replace sidewalks (IMR 13-14) |  |  |

a)	Enrollment Pressures / Learning Opportunities:  none anticipated

| 3 Year Projection |  |  |  |
| --- | --- | --- | --- |
| Year | 15-16 | 16-17 | 17-18 |
| Gr.K-7 | 346 | 342 | 323 |

b)	Program Changes:  none anticipated

c)	Enrollment History

| 8 Year Historical |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | 07-08 | 08-09 | 09-10 | 10-11 | 11-12 | 12-13 | 13-14 | 14-15 |
| Gr.K-7 | 315 | 292 | 281 | 276 | 267 | 293 | 304 | 346 |

| Ecole Mountain View School 141 MacLeod Avenue Hinton Alberta |  |
| --- | --- |
| Grades | K – 7 English K – 7 French Immersion |
| Enrollment (2014/2015) | 192 English, 276 French Immersion |
| Current Utilization (2013/2014) | 75 % |
| Alberta Infrastructure Net Capacity | 610 |
| Type of Construction | Masonry |
| Year Constructed | 1957 – Original building 2515.44 M2 1958, 1962, 1967 – Additions (1957, 1958, 1962 – Additions demolished in 2004) 2004 – Modernization and Addition of two Story Section & Gym – 2406.90 M2 2007 – Addition of 2 Modus modular – 249.03 M2 |
| Total Building Area | 5,171.37 M2 |
| FCI | 2.54 % September 2012 |

Facility Status Summary Chart

| Last Facility Audit (1999) | Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- | --- |
| Facility Audit Outdated |  | 2015 – Repair Stucco |  |
|  | 2004/05 – Complete Modernization & Addition |  |  |
|  | 2006 – Parking Lot paved (IMR) |  |  |
|  | 2008 – 2 Modular Classrooms added |  |  |
|  | 2014 – Flooring gathering area (IMR 13-14) |  |  |
|  | 2014 – Replace paging/bell system (IMR 13-14) |  |  |

- Enrollment Pressures / Learning Opportunities:

| 3 Year Projection |  |  |  |
| --- | --- | --- | --- |
| Year | 15-16 | 16-17 | 17-18 |
| Gr.K-7 | 463 | 469 | 467 |

b)	Program Changes:  none anticipated

c)	Enrollment History

| 8 Year Historical |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | 07-08 | 08-09 | 09-10 | 10-11 | 11-12 | 12-13 | 13-14 | 14-15 |
| Gr.K-7 | 457 | 479 | 465 | 461 | 464 | 470 | 464 | 468 |

| Ecole Pine Grove School 4619 – 12 Avenue Edson Alberta |  |
| --- | --- |
| Grades | 6 – 8 English 6 – 8 French Immersion |
| Enrollment (2014/2015) | 295 English, 37 French Immersion |
| Current Utilization (2013/2014) | 77 % |
| Alberta Infrastructure Net Capacity | 497 |
| Type of Construction | Masonry |
| Year Constructed | 1968 – Original building – 2709.50 M2 1994 – Addition of New Gymnasium – 710.50 M2 2005 – Classroom Addition & Partial Modernization – 1101.45 M2 |
| Total Building Area | 4, 521.45 M2 |
| FCI | 8.43 % November 2013 |

Facility Status Summary Chart

| Last Facility Audit (2013) | Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- | --- |
| 2013 – Boiler relief valves are inadequate. Replace with adequate capacity devices |  |  |  |
|  | 2010 – Pave Parking lot 2011 – Refinish Gym Floor 2013 – Re-roof |  |  |
|  | 2009 – Replace Flooring in computer labs (IMR) 2011 – Replace wood shingles with metal cladding (IMR 11-12) |  |  |
|  | 2005/06 – A Classroom addition & Heating & Ventilation Upgrade was completed |  |  |
|  | 2006 – Upgrade Washrooms (IMR) |  |  |
|  | 2007 – Replace Hallway Flooring (IMR) |  |  |
|  | 2007 – Sidewalk replacement (IMR) 2011 – Replace Sidewalks |  |  |

a)	Enrollment Pressures / Learning Opportunities:  none anticipated

| 3 Year Projection |  |  |  |
| --- | --- | --- | --- |
| Year | 15-16 | 16-17 | 17-18 |
| Gr.6-8 | 341 | 334 | 354 |

b)	Program Changes:  none anticipated

c)	Enrollment History

| 8 Year Historical |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | 07-08 | 08-09 | 09-10 | 10-11 | 11-12 | 12-13 | 13-14 | 14-15 |
| Gr.6-8 | 440 | 421 | 399 | 372 | 353 | 350 | 327 | 332 |

| Ecole Westhaven School 1205 Westhaven Drive Edson Alberta |  |
| --- | --- |
| Grades | K – 5 English K – 5 French Immersion |
| Enrollment (2014/2015) | 197 English, 141 French Immersion |
| Current Utilization (2013/2014) | 71 % |
| Alberta Infrastructure Net Capacity | 422 |
| Type of Construction | Masonry |
| Year Constructed | 1982 – Original building – 3144.62 M2 2003 – Addition of Five Portables - 477.68 M2 |
| Total Building Area | 3, 622.30 M2 |
| FCI | 10.22% October 2012 |

Facility Status Summary Chart

| Last Facility Audit (2007) | Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- | --- |
| 2012 - Joint Sealers – Replace exterior sealants with single component urethane caulk. | 2013 - Painting | 2014 – Re-coat gym floor (IMR 14-15) |  |
| Showers – Repair leak and make shower usable. Check for water damage | 2012 – Repair leaks |  |  |
| 2012 - Water Heaters – Replace both water heaters |  |  |  |
| Washroom Floor Drain – Dig out damaged sections and replace drain | 2009 - Completed |  |  |
| Entrance Sodium Lighting – Replace with metal halide or fluorescent | 2009 - Completed |  |  |
| 2012 - Barrier Free Entrances – Install power assists on main door |  |  |  |
|  | 2001 – New Hardwood Floor installed in Gym (IMR) |  |  |
|  | 2003 – Hot Water Heating Lines replaced (IMR) |  |  |
|  | 2006 – Re-Roofing (IMR) |  |  |
|  | 2006 – Replaced Sidewalk (IMR) 2012 – Replace front sidewalk (IMR 11-12) |  |  |
|  | 2007 – DDC Controls installed (IMR) |  |  |
|  | 2014 – Install cardlock system (IMR 13-14) |  |  |

a)	Enrollment Pressures / Learning Opportunities:

| 3 Year Projection |  |  |  |
| --- | --- | --- | --- |
| Year | 15-16 | 16-17 | 17-18 |
| Gr.K-5 | 356 | 367 | 376 |

b)	Program Changes:  none anticipated

- Enrollment History

| 8 Year Historical |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | 07-08 | 08-09 | 09-10 | 10-11 | 11-12 | 12-13 | 13-14 | 14-15 |
| Gr.K-5 | 312 | 308 | 294 | 264 | 263 | 301 | 315 | 338 |

| Evansview School 4606 – 50 Street Evansburg Alberta |  |
| --- | --- |
| Grades | K - 6 |
| Enrollment (2014/2015) | 125 |
| Current Utilization (2013/2014) | 58 % |
| Alberta Infrastructure Net Capacity | 211 |
| Type of Construction | Masonry, One Storey |
| Year Constructed | 1961 – Original building – 568.56 M2 1963 – North Section Addition – 1005.95 M2 1982 – South Section Addition – 488.30 M2 |
| Total Building Area | 2,062.81 M2 |
| FCI | 23.55 % September 2011 |

Facility Status Summary Chart

| Last Facility Audit (2006) | Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- | --- |
| Slab on Grade 1963 Section – Repair slab | 2009 – Hallway Flooring replacement (IMR) | 2015 – Install DDC (IMR 14-15) |  |
| Wood Windows (Glass & Frame) 1961 & 1963 Section – Replace windows 1963 section | 2007 – Windows replaced (IMR) | 2015 – Install Cardlock (IMR 14-15) |  |
| Built-Up Bituminous Roofing (Asphalt & Gravel) 1982 Section – Replace roof & insulation | All roof sections have been replaced (IMR) | 2015 – Upgrade Washrooms (IMR 14-15) |  |
| Metal Gutters & Down spouts 1982 Section – Replace gutters & down - spouts | Replaced during re-roofing |  |  |
| Interior Windows – Install new frames & GWG glazing |  |  |  |
| Air Handling Units: Air Distribution 1982 Section Offices – Install a new air handling unit |  |  |  |
| Interior Fluorescent Fixtures – Upgraded T12 fixtures to T8 Fixtures |  |  |  |
| Public Address & Music System – Replace existing PA system |  | As required |  |
| Barrier Free Entrances – 1982 Section – Install power assist door operators |  |  |  |
| Site Earthwork (Site Grading) – Provide positive site slope away from building | 2008 – Replace sidewalk & improve drainage |  |  |
| Aggregate Roadway/Parking Lot (Gravel) – Regrade & pave roadway & parking & Install accessories | 2008 – Parking lot paved (IMR) |  |  |
| Flagpoles – Replace flagpoles | 2007 – Flagpoles replaced (IMR) |  |  |
|  | 2006 – Exterior Painted (P.O. & M) |  |  |
|  | 2010 – Refinish Gym Floor 2014 – Install Fire Alarm panel |  |  |

a)	Enrollment Pressures / Learning Opportunities: Declining enrollments

| 3 Year Projection |  |  |  |
| --- | --- | --- | --- |
| Year | 15-16 | 16-17 | 17-18 |
| Gr.K-6 | 120 | 116 | 111 |

b)	Program Changes:  none anticipated

c)	Enrollment History

| 8 Year Historical |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | 07-08 | 08-09 | 09-10 | 10-11 | 11-12 | 12-13 | 13-14 | 14-15 |
| Gr.K-6 | 120 | 92 | 80 | 91 | 110 | 106 | 120 | 125 |

| Harry Collinge High School 158 Sunwapta Drive Hinton Alberta |  |
| --- | --- |
| Grades | 8 – 12 English 8 – 12 French Immersion |
| Enrollment (2014/2015) | 376 English, 54 French Immersion, Unfunded 1 |
| Current Utilization (2013/2014) | 48 % |
| Alberta Infrastructure Net Capacity | 992 |
| Type of Construction | Masonry |
| Year Constructed | 1958 – Original building – 412.2 M2 1960, 1963, 1969 – Additions – 6,844.60 M2 1988 – Gymnasium Addition – 1695.37 M2 1989 / 92 – Modernization 2009 – Partial Modernization & HVAC Upgrade |
| Total Building Area | 8, 952.17 M2 |
| FCI | 8.08 % September 2012 |

Facility Status Summary Chart

| Last Facility Audit (2007) | Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- | --- |
| PA and Music Systems – Replace with new system | 2009 - Completed | 2015 – Re-roof (IMR 14-15) |  |
|  | 2007 – Re Roofing (IMR) 2010 – Re-roofing RA12 2011 – Re-roof RA 8 | 2015 – Install Cardlock (IMR 14-15) |  |
|  | 2008 – Re Roofing (IMR) | 2015 – Replace Fire Alarm Panel (IMR 14-15) |  |
|  | 2008 – Replace Flooring in Cafeteria (IMR) 2010 – Replace hallway flooring 2014 – Replace flooring-Art room, Principal office & music room (IMR 13-14) 2012 – Repair Boiler (IMR 12-13) 2013 – Painting 2013 - Flooring |  |  |
|  | 2008 – Refinish Gym Floor (IMR) 2012 – Re-roof RA 4,4A,4B (IMR 11-12) 2014 – Re-roof RA 9 (IMR 13-14) |  |  |
| Science Labs and IA shop – Install electronic exhaust fans | 2008/09 – Upgrade Heating & Ventilation & Classrooms in 1969 Section |  |  |
| Skylight Assembly – Convert to Clearstorey. 9 Acrylic Skylights - Replace | 2008 – Replace Skylight with Clear Storey (IMR) |  |  |
|  | 2010 – Security Cameras installed |  |  |

- Enrollment Pressures / Learning Opportunities:  none anticipated

| 3 Year Projection |  |  |  |
| --- | --- | --- | --- |
| Year | 15-16 | 16-17 | 17-18 |
| Gr.8-12 | 416 | 407 | 420 |

- Program Changes:  none anticipated

- Enrollment History

| 8 Year Historical |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | 07-08 | 08-09 | 09-10 | 10-11 | 11-12 | 12-13 | 13-14 | 14-15 |
| Gr.8-12 | 580 | 537 | 543 | 481 | 447 | 464 | 442 | 430 |

| Grande Cache Community High School 10601 Shand Avenue Grande Cache Alberta |  |
| --- | --- |
| Grades | K – 12 |
| Enrollment (2014/2015) | 290, English |
| Current Utilization (2013/2014) | 65 % |
| Alberta Infrastructure Net Capacity | 535 |
| Type of Construction | Masonry |
| Year Constructed | 1971 – Original building – 4,817.30 M2 1989 – Addition of Elite Shop – 743.80 M2 |
| Total Building Area | 5,561.10 M2 |
| FCI | 25.92 % October 2010 |

Facility Status Summary Chart

| Last Facility Audit (2010) | Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- | --- |
| 2010 - Air Quality – Provide range hoods with adequate exhaust and a roof mounted make up air unit |  | 2015 – Replace Fence (IMR 14-15) | This facility was approved for modernization in 2014 |
| 2010 - Detection & Fire Alarm – Replace fire alarm system with new system designed to comply with current code |  |  |  |
| 2010 - Brick Masonry – Loose & falling bricks. Repair brick facade |  |  |  |
| 2010 - Floor Construction Fire stopping – Install fire stopping at deficient locations. Install appropriate fire rated assembly to form void of Double Tee Structures | 2013 Painting 2013 - Flooring |  |  |
|  | 2009 – Gym Floor Replacement & Re-Finish (IMR) 2011 – Replace rooftop unit |  |  |
|  | 2007 – Re Roofing (IMR) 2008 – Re Roofing (IMR) 2009 – Re Roofing (IMR) 2010 – Re-Roof RA 2 2011 – Re-Roof RA 1 Gym 2014 – RA 5 Insurance claim (IMR 13-14) |  |  |
|  | 2011 – Install handicap accessible sidewalk(IMR 10-11) 2012 Install new sidewalk (IMR 11-12) |  |  |
|  | 2007 – New Flooring in Hallways & Classrooms (P.O. & M) 2010 – Replace hallway flooring |  |  |

- Enrollment Pressures / Learning Opportunities: None anticipated.

| 3 Year Projection |  |  |  |
| --- | --- | --- | --- |
| Year | 15-16 | 16-17 | 17-18 |
| Gr.K-12 | 287 | 301 | 294 |

b)	Program Changes:  none anticipated

- Enrollment History

| 8 Year Historical |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | 07-08 | 08-09 | 09-10 | 10-11 | 11-12 | 12-13 | 13-14 | 14-15 |
| Gr.K-12 | 286 | 341 | 323 | 316 | 348 | 346 | 317 | 290 |

| Jasper Elementary School 300 Elm Street Jasper Alberta |  |
| --- | --- |
| Grades | K – 6 English K – 6 French Immersion |
| Enrollment (2014/2015) | 88 English, 131 French Immersion |
| Current Utilization (2013/2014) | 63 % |
| Alberta Infrastructure Net Capacity | 368 |
| Type of Construction | Masonry |
| Year Constructed | 1964 – Original building – 2,940.00 M2 1998 – Modernization – 289.40 M2 |
| Total Building Area | 3, 229.40 M2 |
| FCI | 5.48 % September 2013 |

Facility Status Summary Chart

| Last Facility Audit (2013) | Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- | --- |
| 2013 – Floor Fire Separations in upper ceiling spaces need to be re-sealed with fire rated products |  |  |  |
| 2013 – Replace deteriorated hot & cold water supply piping |  |  |  |
| 2013 – Replace 3 main shut off valves to allow complete shut off of domestic water |  |  |  |
| 2013 – Install Backflow Preventer in irrigation system |  |  |  |
| 2013 – Replace all hose bibs on exterior of school |  |  |  |
| 2013 – Replace pipe insulation on all domestic water lines after lines have been replaced |  |  |  |
| 2013 – Add and adjust supports to gas lines as needed |  |  |  |
| 2013 – Add emergency lights where levels are below 1 fc |  |  |  |
| 2008 – Remove Water Storage Tanks |  | 2015 – Replace Boiler (IMR 14-15) |  |
| 2008 – Replace exterior hose bibbs | 2014 – Install cardlock access (IMR 13-14) |  |  |
| 2008 – Replace Metal Gutters & Downspouts | 2013 – Refinish Gym Floor (IMR 12-13) |  |  |
| Facility Audit Outdated | 2013 - Painting |  |  |
|  | 1998 – Entire Facility Renovated |  |  |
|  | 2006 – Replace Tile in Staffroom & Washrooms (IMR) |  |  |

a)	Enrollment Pressures / Learning Opportunities:  none anticipated

| 3 Year Projection |  |  |  |
| --- | --- | --- | --- |
| Year | 15-16 | 16-17 | 17-18 |
| Gr.K-6 | 211 | 180 | 174 |

b)	Program Changes:  none anticipated

c)	Enrollment History

| 8 Year Historical |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | 07-08 | 08-09 | 09-10 | 10-11 | 11-12 | 12-13 | 13-14 | 14-15 |
| Gr.K-6 | 245 | 231 | 233 | 247 | 233 | 230 | 241 | 219 |

| Jasper Jr. / Sr. High School 401 Bonhomme Street Jasper Alberta |  |
| --- | --- |
| Grades Verify French Immersion grade level | 7- 12 English 7-11 French Immersion |
| Enrollment (2014/2015) | 112 English, 90 French Immersion |
| Current Utilization (2014/2015) | N/A |
| Alberta Infrastructure Net Capacity | 375 |
| Type of Construction | Steel Frame/Masonry |
| Year Constructed | 2014 – Original building |
| Total Building Area | 6,662 M2 |
| FCI |  |

Facility Status Summary Chart

| Last Facility Audit () | Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- | --- |

- Enrollment Pressures / Learning Opportunities:  none anticipated

| 3 Year Projection |  |  |  |
| --- | --- | --- | --- |
| Year | 15-16 | 16-17 | 17-18 |
| Gr.7-12 | 209 | 212 | 202 |

- Program Changes:  none anticipated

- Enrollment History

| 8 Year Historical |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | 07-08 | 08-09 | 09-10 | 10-11 | 11-12 | 12-13 | 13-14 | 14-15 |
| Gr.7-12 | 262 | 244 | 242 | 231 | 247 | 210 | 194 | 202 |

| Parkland Composite High School 4630 - 12 Avenue Edson Alberta |  |
| --- | --- |
| Grades | 9 – 12 English 9 – 12 French Immersion |
| Enrollment (2014/2015) | 474 English, 36 French Immersion |
| Current Utilization (2013/2014) | 75 % |
| Alberta Infrastructure Net Capacity | 894 (does not include 1 modular) |
| Type of Construction | Masonry |
| Year Constructed | 1968 – Original building – 6,887.50 M2 1994 – Gymnasium – 697 M2 2004 / 05 – Addition & Modernization – 448 M2 2008 – Addition of 4 Modus Modulars – 438.48 M2 |
| Total Building Area | 8,470.98 M2 |
| FCI | 0.23 % November 2013 |

Facility Status Summary Chart

| Last Facility Audit (2013) | Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- | --- |
| 2013 – Emergency Power System. Generator is beyond useful life, spare parts are hard to find. |  | # 2015 – Insulate music room wall |  |
|  | 2010 – Pave Parking Lot |  |  |
|  | 2005/06 – Complete Modernization |  |  |
|  | 2005 – Gym Floor repaired & refinished (IMR) |  |  |
|  | 2009 – Nine Portable demolished |  |  |
|  | 2009 – 4 New Modulars added |  |  |
|  | 2009 New Cardlock Access Installed |  |  |
|  | 2013 – Flooring 2014 – Flooring room 110, Art room, Meeting room & 3 offices (IMR 13-14) |  |  |
|  | 2013 – Painting |  |  |
|  | 2013 – Re-roof (IMR 12-13) 2013 – Install Concrete pad (IMR 12-13) 2013 – Replace Hot Water Tank (IMR 12-13) |  |  |
|  | 2014 – Install railing in Gym |  |  |

- Enrollment Pressures / Learning Opportunities:  none anticipated

| 3 Year Projection |  |  |  |
| --- | --- | --- | --- |
| Year | 15-16 | 16-17 | 17-18 |
| Gr.9-12 | 485 | 510 | 481 |

- Program Changes:  none anticipated

- Enrollment History

| 8 Year Historical |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | 07-08 | 08-09 | 09-10 | 10-11 | 11-12 | 12-13 | 13-14 | 14-15 |
| Gr.9-12 | 570 | 565 | 563 | 576 | 546 | 556 | 552 | 510 |

| Sheldon Coates School 11080 Swann Drive Grande Cache Alberta |  |
| --- | --- |
| Grades | K - 3 |
| Enrollment (2014/2015) | 159 |
| Current Utilization (2013/2014) | 70 % |
| Alberta Infrastructure Net Capacity | 217 (does not include 1 modular) |
| Type of Construction | Masonry |
| Year Constructed | 1976 – Original building – 1,913.50 M2 2007 – Addition of 1 Modus Modular – 230.96 M2 2008 – Addition of 1 Modus Modular – 111.48 M2 |
| Total Building Area | 2, 255.94 M2 |
| FCI | 30.06 % September 2012 |

Facility Status Summary Chart

| Last Facility Audit (2012) | Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- | --- |
| 2012 – Barrier Free – Install Power Assist Doors |  | 2015 – Install lockers (IMR 14-15) |  |
| 2012 - Window and Door Sealants – Replace with new backer rod and urethane caulking | 2013 - Painting | 2015 – Upgrade library lighting (IMR 14-15) |  |
| Exterior Gym Wall - Repaint | 2010 – Metal cladding installed |  |  |
| PA and Music System – Replace system |  |  |  |
| Barrier Free Entrance – Install power assist doors at main entry |  |  |  |
|  | 2009 - New Modular |  |  |
|  | 2006 – Replaced Heat Pumps (IMR) |  |  |
|  | 2006 – Re-Roofing (IMR) |  |  |
|  | 2007 – Re-Roofing (IMR) |  |  |
|  | 2009 – Re-Roofing (IMR) 2014 – Replace siding with Metal Cladding (IMR13-14) |  |  |
|  | 2007 – New Flooring in Classrooms & Library (IMR) |  |  |
| Replace BUR – Re-Roofing over west classrooms | 2008 – Re-Roofing (IMR) |  |  |
|  | 2007 – 1 New Modular |  |  |
|  | 2008 - 1 New Modular |  |  |
|  | 2009 – 1 new modular |  |  |
|  | 2012 - Repair broken water line (IMR 11-12) |  |  |

- Enrollment Pressures / Learning Opportunities:

Expected growth in this area could result in higher enrollments.

| 3 Year Projection |  |  |  |
| --- | --- | --- | --- |
| Year | 15-16 | 16-17 | 17-18 |
| Gr.K-3 | 159 | 2153 | 147 |

b)	Program Changes:  none anticipated

c)	Enrollment History

| 8 Year Historical |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | 07-08 | 08-09 | 09-10 | 10-11 | 11-12 | 12-13 | 13-14 | 14-15 |
| Gr.K-3 | 176 | 155 | 165 | 179 | 173 | 190 | 187 | 159 |

Non School Buildings

Maintenance Shops

| Wildwood Shop |  |
| --- | --- |
| Original Construction | 1956 |
| Modernization | 1985 |

The Wildwood shop is the old gymnasium that was detached from the school during the modernization in 1985. The shop was re-roofed in 2007 and new heating was installed.

| Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- |
| 1985 - Converted old gymnasium that was detached from the school during modernization |  |  |
| 2007 – Re-roofed |  |  |
| 2007 – New heating installed |  |  |
|  | 2011 – None at this time |  |

| Hinton Shop |  |
| --- | --- |
| Original Construction | 1989 |

The Hinton shop is a 2 car garage that was re-roofed in 2010. The shop is undersized and needs to be expanded.

| Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- |
| 1989 - Constructed |  | Expand shop |
| 2010 – Re-roofed |  |  |

| Grande Cache Shop |  |
| --- | --- |
| Original Construction | 1996 |

The Grande Cache shop is attached to Sheldon Coates School

| Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- |
| 1996 - Constructed |  |  |
|  | No Immediate Needs |  |

| Bus Compound |  |
| --- | --- |
| Purchased | February 2009 |

This compound was purchased to store Transportation busses and Maintenance/Technology vehicles for the Hinton area.

| Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- |
| July 2009 – Electric gate installed |  |  |

| Educational Services Centre |  |
| --- | --- |
| Original Construction | Unknown |
| Modernization | 1988 |

This building houses all Administration, Facilities, Transportation, and Technology.

| Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- |
| Board Room – New wall paper and paint |  | 5 – 10 Years - Heating and Ventilation |
| 2009 - Main Floor Reception/Washrooms – New flooring, millwork, bathroom partitions |  | 5 – 10 Years – Re-Roof |
| 2011 - Staff room renovation |  |  |

| Student Support Services Centre |  |
| --- | --- |
| Original Construction | 2006-2007 |

Four portables and addition were moved onto the A.H.Dakin site in 2006-2007. This building houses the Edson Playschool and Learning Services Support personnel.

| Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- |
| 2006-2007 – Moved 4 portables and addition onto A.H. Dakin site |  |  |
|  | None at this time |  |

| Hinton Learning Services Centre |  |
| --- | --- |
| Original Construction | 1964 |

This building houses TLC Hinton and Hinton Learning Services

| Work Completed | Planned Upgrading | Expected Capital Project in Next 10 yrs |
| --- | --- | --- |
| 2014-2015 – Complete renovation |  |  |